Name:vrajesh

Q1. List the iP address cLasses (a,

B, c, d, e) and give one reaL-worLd examPLe of a device or service

that might use each cLass. write your answers in a taBLe format.

answer:

cLass iP address range  defauLt suBnet mask examPLe a 1.0.0.0 – 126.255.255.255

255.0.0.0 Large comPany network

B 128.0.0.0 – 191.255.255.255

255.255.0.0 university or coLLege network

c 192.0.0.0 – 223.255.255.255

255.255.255.0 home wifi router d 224.0.0.0 – 239.255.255.255 not used for hosts video streaming or muLticast services

e 240.0.0.0 – 255.255.255.255

reserved research and testing PurPoses

Q2. given the iP address

192.168.10.45 with suBnet mask

255.255.255.0, caLcuLate the network address, Broadcast

address, and the range of vaLid host addresses.

answer:

given: iP address = 192.168.10.45

suBnet mask = 255.255.255.0 (/24)

steP 1: find network address

network address = iP address and suBnet mask

192.168.10.45 and 255.255.255.0 = 192.168.10.0

steP 2: find Broadcast address

since suBnet mask is /24, the Last octet is for hosts.

Broadcast address =

192.168.10.255

steP 3: find vaLid host range

first host = 192.168.10.1

Last host = 192.168.10.254 finaL answer

item address

network address 192.168.10.0

Broadcast address

192.168.10.255 first host 192.168.10.1

Last host 192.168.10.254

Q3. create a suBnetting PLan for a smaLL food deLivery startuP Like

Zomato that needs 4 seParate suBnets: one for deLivery

Partners, one for restaurants, one for customers, and one for admins. assign a uniQue suBnet

(with network address and suBnet mask) for each grouP using the

10.0.0.0/24 network.

answer:

the network 10.0.0.0/24 can Be

divided into 4 eQuaL suBnets.

suBnet mask = 255.255.255.192 (/26)

grouP network address suBnet mask

deLivery Partners 10.0.0.0/26

255.255.255.192 restaurants 10.0.0.64/26

255.255.255.192 customers 10.0.0.128/26

255.255.255.192 admins 10.0.0.192/26

255.255.255.192

exPLanation:

each suBnet Provides 62 usaBLe host addresses.

Q4. you are given the foLLowing scenario: a coLLege wifi network

uses the iP range 172.16.0.0/16. the admin wants to create at Least 20 suBnets for different BuiLdings.

caLcuLate the new suBnet mask and the numBer of hosts avaiLaBLe Per suBnet.

answer:

given:

network = 172.16.0.0/16

need = at Least 20 suBnets

steP 1: Borrow Bits

find numBer of Bits needed:

- = 16 suBnets ❌

- = 32 suBnets ✅

Borrow 5 Bits

steP 2: new Prefix

originaL Prefix = /16

new Prefix = 16 + 5 = /21

steP 3: new suBnet mask

/21 = 255.255.248.0

steP 4: caLcuLate hosts Per suBnet

host Bits = 32 − 21

= 11 Bits

hosts = 2¹¹ − 2

= 2048 − 2

= 2046 hosts

finaL answer

item vaLue reQuired suBnets 20 Borrowed Bits 5 new Prefix /21

new suBnet mask 255.255.248.0 numBer of suBnets 32

hosts Per suBnet 2046

Q5. a friend sends you a message asking why their device with iP

192.168.1.300 is not connecting to the network. write a short

exPLanation identifying the error and what a vaLid iP address in that suBnet shouLd Look Like. answer:

the iP address 192.168.1.300 is invaLid Because an iP address

octet can onLy have vaLues from 0 to 255.

here, 300 is greater than 255, so

the device cannot connect to the network.

a vaLid iP address in this suBnet shouLd Look Like:

192.168.1.10

192.168.1.50

192.168.1.100

192.168.1.200 short exPLanation:

"your iP address is incorrect

Because the Last numBer is 300,

But iP address vaLues can onLy Be Between 0 and 255. use an address such as 192.168.1.100 or another numBer Between 1 and 254 in the

Last section of the iP address."